# HP-200 Torque and Fluid Specifications

Document ID: MM-HP-200-TS

Revision: 2

Owner: Maintenance Planner

## Torque Specifications

Use a calibrated torque wrench (see QCS-004). Lubricate threads with clean ISO VG 46 oil unless the note says otherwise. Apply LOTO per SOP-SAF-001 before working on the press.

| Fastener | Location | Size | Torque (ft-lb) | Pattern / note |
| --- | --- | --- | --- | --- |
| Gland retainer bolts | Main cylinder | M20 | 210 | Star pattern, 2 passes |
| Cylinder mounting bolts | Crown | M30 | 650 | Star pattern, 3 passes |
| Platen clamp bolts | Moving platen | M24 | 380 | Cross pattern |
| Guide post nuts | Columns | M64 | 1,450 | Hydraulic tensioner only |
| Pump coupling set screws | Power unit | M10 | 35 | Blue thread locker |
| Accumulator mounting U-bolts | Power unit | 1/2 in | 60 | Re-check after 24 h |

## Fluid Specifications

| Fluid | Specification | Capacity | Change interval |
| --- | --- | --- | --- |
| Hydraulic oil | ISO VG 46 anti-wear | 120 gal | 4,000 h or annually |
| Guide post grease | EP-2 lithium | - | Monthly |
| Accumulator gas | Dry nitrogen | 1,400 psi pre-charge | Check monthly |

## Escalation

Contact the Maintenance Planner (ext. 4410) for any fastener not listed here.